# 5311F	FORMAN SUSPENDED TILE CEILING SYSTEMS

## 1.	GENERAL

This section relates to the supply and installation of Forman Building Systems suspended tile ceiling systems, including;

-	Suspension system

-	Ceiling tiles

-	Grid

-	Perimeter trim profile

-	All elements offered by the manufacturer to complete the system

### 1.1	RELATED WORK

Refer to ~ for ~

### 1.2	ABBREVIATIONS AND DEFINITIONS

Refer to the general section 1232 INTERPRETATION & DEFINITIONS for abbreviations and definitions used throughout the specification.

The following abbreviations apply specifically to this section:

FRR	Fire resistance rating

NRC	Noise reduction coefficient

CAC	Ceiling attenuation class

STC	Sound transmission class

Documents

### 1.3	DOCUMENTS

Refer to the general section 1233 REFERENCED DOCUMENTS.  The following documents are specifically referred to in this section:

NZBC C/AS1	Fire safety

AS/NZS 1170.1	Structural design actions - Permanent, imposed and other actions

NZS 1170.5	Structural design actions - Earthquake actions - New Zealand

AS/NZS 2785	Suspended ceilings - Design and installation

NZS 4219	Seismic performance of engineering systems in buildings

AS 2946	Suspended ceilings, recessed luminaires and air diffusers - Interface requirements for physical compatibility

ASTM C423	Test method for sound absorption and sound absorption coefficients by the reverberation room method

ASTM E1414	Standard test method for airborne sound attenuation between rooms sharing a common ceiling plenum (two room method)

### 1.4	MANUFACTURER/SUPPLIER DOCUMENTS

Manufacturer's and supplier's documents relating to this part of the work:

Armstrong:	Including grid systems and accessories, 30 year warranty, installation, maintenance, safety data sheets, Green Star Assessment, Environmental Choice.

Armstrong Humiguard:

Ceiling tile range including RH95, RH99 and RH100 tiles

Nittobo:	Solaton T&G staple and adhesive

Eurocoustic:	Ceiling tile range

Parafon:	Ceiling tile range including grid and direct fix.

Climateline:	powdercoated and hygiene plasterboard panels

Boral:	Echostop perforated plaster

Gyprock:	Perforated Plasterboard

GibTone:	Suspended Ceiling Tiles (Plain, Vinyl)

Fenta:	Ceiling Panels

Renhurst:	Ripplesound, rippletone acoustic metal ceilings

ICE:	Metal Ceiling Panels

Coruline:	Linear Metal Ceilings

ACS:	Linear 5 Metal Ceilings

Lindner:	Metal Ceilings

Armstrong:	Metalworks Metal Ceilings

Rondo:	Ceiling grid and Suspension Systems

Access Panels Slimline and Panther

Novahush:	Baffle Stack Inter-office Sound Control Systems

Firefly:	Fire and smoke Barriers, and accessories

BRANZ Opinion FAR 1627 Firefly Canopies in Fire Resistance Floor Systems

BRANZ Opinion FAR 2701 Resistance of Firefly Phoenix Cavity Fire Barrier

BRANZ Opinion FAR 1628 Fire Resistance of Firefly Plus 60 Cavity Fire Barrier

Manufacturer/supplier contact details

Company:	Forman Building Systems Ltd

Web:	www.forman.co.nz/products.php

Telephone:	0800 45 4000

Requirements

### 1.5	NO SUBSTITUTIONS

Substitutions are not permitted to any specified system, or associated components and products.

### 1.6	SAMPLE SECTION

Allow to erect a sample section of the suspended ceiling system offered.  Subject to confirmation in writing, the sample section may form part of the completed installation.

Refer to SELECTIONS for location of sample section.

### 1.7	INSTALLATION

To AS/NZS 2785.  Installation by a manufacturer's accredited installer, using the manufacturer's technical services.  Provide evidence of experience, listing completed projects of similar size and complexity.  Installation of Firefly Cavity Barriers must be by trained installers.

Installation to comply with the loadings code AS/NZS 1170.1.  Installation to comply with the requirements of NZS 4219; with related building services installations complying specifically with clauses 5.9 Ducting, 5.14 Luminaires and 5.13 Ceiling-suspended equipment and equipment in ceiling voids.

### 1.8	CLEANING INSTRUCTIONS

Supply information on the materials and method of cleaning the ceiling system over its expected life.

### 1.9	SPARES

Provide spare matching ceiling elements in the quantities specified in SELECTIONS.  Deliver into a dry store at the site or elsewhere as directed and at agreed times.

Performance

### 1.10	LOADING CODE REQUIREMENT

Comply with the requirements of AS/NZS 1170.1 or NZS 1170.5.

### 1.11	CERTIFICATION

Provide:

-	certification of compliance with NZS 1170.5, section 8 for evaluation

-	certificates and other evidence that the system offered complies with the standards of performance specified

-	a Producer Statement on completion.

### 1.12	ACOUSTIC REQUIREMENTS

Refer to SELECTIONS for requirements.  Use an independent testing authority to test a specimen of the ceiling system to ASTM C423 and ASTM E1414, and submit the results for acceptance.

### 1.13	EARLY FIRE HAZARD PROPERTIES

Early fire hazard properties of internal surface materials/systems when tested to NZBC C/AS1, Appendix C, Test Methods, must not exceed the following indices to NZBC C/AS1, Table 6.2.

| Properties | Index | Item | Location |
| --- | --- | --- | --- |
| (FI) Flammability index | ~ | Suspended fabrics | ~ |
| (SFI) Spread of flame index | 0 | Ceilings | Exitways |
| (SFI) Spread of flame index | ~ | ~ | ~ |
| (SDI) Smoke development index | 3 | Ceilings | Exitways |
| (SDI) Smoke development index | ~ | ~ | ~ |

### 1.14	FIRE RATING REQUIREMENT

Design the ceiling system, including cavity fire and smoke barriers where required, so that together with the floor it is to NZBC C/AS1: Part 5 Fire resistance ratings.  Refer to SELECTIONS for requirements.

### 1.15	ENVIRONMENTAL REQUIREMENTS

Design the ceiling system for use over its expected life without deterioration.  Refer to SELECTIONS for requirements.

### 1.16	REFLECTANCE

Refer to SELECTIONS for requirements.

## 2.	PRODUCTS

Materials - Armstrong two way exposed grid system

### 2.1	ARMSTRONG PEAKFORM SUSPENSION SYSTEM

Grid finish/colour:	Armstrong global white

### 2.2	PERIMETER TRIM - STEEL

Brand/form:	Armstrong PeakForm perimeter trim by Forman Building Systems

Material:	Hot-dip galvanized steel

Finish/colour:	Armstrong global white

### 2.3	ARMSTRONG BLUE TONGUE ALUMINIUM SUSPENSION SYSTEM

Grid finish/colour:	Armstrong global white or Dulux Satin White.

### 2.4	PERIMETER TRIM - ALUMINIUM

Brand/form:	Armstrong AXIOM aluminium perimeter trim

Material:	Extruded aluminium

Finish/colour:	Armstrong global white

### 2.5	PERIMETER PELMET/BLIND BOXES

Brand/form:	Armstrong AXIOM Pelmet and Blind Boxes

Material:	Extruded aluminium

Finish/colour:	Armstrong global white

### 2.6	CEILING TILES

Supplied by Forman Building Systems.

Materials - Rondo Duo two way exposed suspended ceiling grid

### 2.7	RONDO DUO SUSPENSION SYSTEM

Grid finish/colour:	Rondo standard white

### 2.8	PERIMETER TRIM

Brand/form:	Rondo Duo perimeter trim by Forman Building Systems,

Freephone: 0800 45 4000.

Material:	Hot-dip galvanized steel

Finish/colour:	Rondo white

Materials - Renhurst metal acoustic suspended ceilings

### 2.9	RENHURST METAL SUSPENDED CEILINGS

Refer to SELECTIONS for product selections.

Materials - Coruline linear metal ceilings

### 2.10	CORULINE METAL LINEAR CEILINGS

Refer to SELECTIONS for product selections.

Materials - Linear metal ceilings

### 2.11	LINEAR METAL CEILINGS

Refer to SELECTIONS for product selections.

Materials - Lindner metal ceilings

### 2.12	LINDNER METAL CEILINGS

Refer to SELECTIONS for product selections.

Materials - above ceiling acoustic baffles (non-fire rated)

### 2.13	ABOVE CEILING ACOUSTIC BAFFLES

System:	Forman Baffle Stack inter-office sound control system

CAC rating 46:	600mm wide above Armstrong Humiguard Plus (RH99) ceiling

CAC rating 50:	600mm wide compressed 30% above Armstrong Humiguard Plus (RH99) ceiling

Baffle Stack must not rest directly upon the ceiling tile.

Materials - above ceiling cavity fire barriers

### 2.14	ABOVE CEILING CAVITY FIRE BARRIERS

System:	Forman Firefly cavity fire barriers made from non-respirable glass textile, fire tested with full penetration systems

-/60/60	Forman Firefly Titan fire rated cavity barrier fixed with Rondo steel stud and Keylock wall angle, in accordance with the manufacturer's instructions.

-/60/38	Forman Firefly Plus 60 fire rated cavity barrier fixed with Rondo steel stud and Keylock wall angle, in accordance with the manufacturer's instructions.  Maximum drop 6.1 metres

-/60/30	Forman Firefly Plus 30 fire rated cavity barrier fixed with Rondo steel stud and Keylock wall angle, in accordance with the manufacturer's instructions.  Maximum drop 6.1 metres

-/90/-	Forman Firefly Phoenix smoke and fire barrier fixed with Rondo steel stud and Keylock wall angle, in accordance with the manufacturer's instructions.  Maximum drop 6.1 metres

Firefly cavity smoke, flame and fire barriers must be installed by trained installers using proprietary Firefly Collars, Firefly Shield and Firefly Adhesive.

Components

### 2.15	SCREWS

25mm x 6 gauge non-rusting Type S self-drilling and self-tapping buglehead screws.

## 3.	EXECUTION

Conditions

### 3.1	CO-ORDINATE SERVICES

Co-ordinate and co-operate with electrical and mechanical work to avoid conflict between suspension members and luminaires, diffusers, pipework and ducting.  Confirm the provision of extra hangers and fixings.

Ensure co-operation with work in and above the ceiling, including the marking of specific ceiling tiles below major access points to above-ceiling services.  Colour coded markings to follow the standards laid down by mechanical and electrical services.

### 3.2	SITE CONDITIONS

Do not begin installation until the building is closed in, fully glazed, the roof watertight, and mechanical and electrical duct work above the ceiling completed.

### 3.3	COMPLY

Comply with AS 2946 for interface requirements for physical compatibility.

### 3.4	RESPONSIBILITY

Ensure that conditions are suitable for the ceiling installation.  Arrange for the programming of the work to suit required practice.

Application

### 3.5	INSTALL

Install the system to AS/NZS 2785 minimum standards and the ceiling manufacturer's requirements.

### 3.6	INSTALL ABOVE CEILING ACOUSTIC BAFFLES

Install Baffle Stack with enough compression to ensure stability between slab, roof, or floor above.  No gaps allowed through the width of the baffle.

### 3.7	INSTALL ABOVE CEILING CAVITY FIRE BARRIERS

Install Firefly Cavity Fire Barriers in accordance with the manufacturer's installation manual and BRANZ opinions.

### 3.8	ACCESSIBILITY

Provide access to the ceiling system and the in-ceiling and above-ceiling services so that maintenance and removal of any part can be carried out without damage to the ceiling system or panels.

### 3.9	PENETRATIONS

Accommodate recessed light fittings, air conditioning outlets and other electrical and/or mechanical services that are fixed to or pass through the ceiling system.  Provide independent support for these as necessary.

### 3.10	RETURN AIR PLENUM

Use tiles that do not release fibres into the ceiling space, air conditioning or ventilation system.  Clip down tiles the grid to stop lifting.

### 3.11	PROTECT EXISTING WORK

Protect adjacent existing work from damage during the installation.

Completion

### 3.12	REPLACE

Replace damaged or marked elements.

### 3.13	LEAVE

Leave work to the standard required by following procedures.

### 3.14	REMOVE

Remove debris, unused elements and elements from the site.

### 3.15	CLEAN

Clean soiled or marked units.

## 4.	SELECTIONS

For further details on selections go to www.forman.co.nz/products.php

Substitutions are not permitted to the following, unless stated otherwise.

### 4.1	SAMPLE SECTION

Location:	~

### 4.2	SPARES

Packs:	~

### 4.3	PERFORMANCE REQUIREMENTS

Acoustic

NRC:	~ minimum

Attenuation Class:	~ minimum room to room

Fire

Location:	~

FRR:	~ minimum

Cavity barrier:	~

Floor system:	~

Environmental

Range:	~ - ~°C

Relative humidity:	~ % maximum

Light reflectance

Reflectance:	~ % minimum

For (colour):	~

Recycled content

Percentage:	~ % minimum

Grid and tile system

### 4.4	ARMSTRONG PEAKFORM SUSPENSION SYSTEM

Module size:	~mm x ~mm

Face dimension:	~mm

Perimeter:	~ wall angle

### 4.5	ARMSTRONG BOWMAN SUSPENSION SYSTEM

Module size:	~mm x ~mm

Face dimension:	~mm

Pelmet:	~

### 4.6	ARMSTRONG AXIOM PERIMETER TRIM

Profile:	~

Trim dimension:	~mm

### 4.7	RONDO DUO SUSPENSION SYSTEM

Module size:	~mm x ~mm

Face dimension:	~mm

Perimeter:	~ wall angle

### 4.8	CEILING TILES

Brand:	~

Type:	~

Pattern:	~

Thickness:	~mm

Size:	~mm x ~mm to suit grid

Edge profile:	~

### 4.9	RENHURST METAL SUSPENDED CEILINGS

Panel type:	~

Colour:	~

### 4.10	CORULINE LINEAR METAL CEILINGS

Ceiling type:	~

Colour:	~

### 4.11	ACS LINEAR METAL CEILINGS

Ceiling type:	~

Colour:	~

### 4.12	LINDNER METAL CEILINGS

Ceiling type:	~

Colour:	~

### 4.13	ARMSTRONG METALWORKS METAL CEILINGS

Ceiling type:	~

Colour:	~

### 4.14	ARMSTRONG METALWORKS MESH CEILING

Ceiling type:	Layin Ceilings, 1200mm x 600mm

Mesh type:	~, ~mm

Colour:	~

### 4.15	ARMSTRONG METALWORKS OPEN CELL SYSTEM

Ceiling type:	~mm x ~mm cells

Colour:	~

Grid system:	14mm Suprafine

Panel size:	600mm x 600mm

### 4.16	PERFORATED PLASTERBOARD SHEET

Brand:	Boral Perforated Plasterboard sheet

Thickness:	13mm

Type:	~

### 4.17	ACCESS PANELS

Rondo Panther access panels.

FRR:	~

Acoustic rating:	~

Finish/colour:	~

Size:	~mm x ~mm

Edge:	Standard

Lock:	Budget lock