# 4711F	FORMAN KINGSPAN KOOLTHERM K10FM SOFFIT BOARD

## 1.	GENERAL

This section relates to Kingspan Kooltherm® K10 FM Soffit Board insulation for structural ceiling (soffits);

-	fixed directly to concrete soffits

-	fixed to timber battens

### 1.1	RELATED WORK

Refer to ~ for ~

Documents

### 1.2	DOCUMENTS

Refer to the general section 1233 REFERENCED DOCUMENTS.  The following documents are specifically referred to in this section:

NZBC H1/AS1	Energy efficiency

NZS 4218:2004	Energy efficiency - Small building envelope

NZS 4243	Energy efficiency - Large buildings - Building thermal envelope

AS/NZS 4859.1	Materials for the thermal insulation of buildings - General criteria and technical provisions

BS EN ISO 9001	Quality management systems - Requirements - Technical Corrigendum 1

### 1.3	MANUFACTURER/SUPPLIER DOCUMENTS

Manufacturer’s and supplier’s documents relating to this part of the work:

Kingspan Kooltherm® K10 FM Soffit Board brochure

Manufacturer/supplier contact details

Company:	Forman Building Systems

Web:	www.forman.co.nz

Email:	sales@forman.co.nz

Telephone:	0800 45 4000

Warranties

### 1.4	WARRANTY - MANUFACTURER/SUPPLIER

Provide a material manufacturer/supplier warranty:

10 years:	For Kingspan Kooltherm® K10 FM Soffit Board

-	Provide this warranty on the manufacturer/supplier standard form.

-	Commence the warranty from the date of practical completion of the contract works.

Refer to the general section 1237 WARRANTIES for additional requirements.

### 1.5	WARRANTY - INSTALLER/APPLICATOR

Provide an installer/applicator warranty:

5 years: 	For Kingspan Kooltherm® K10 FM Soffit Board installation

-	Provide this warranty on the installer/applicator standard form.

-	Commence the warranty from the date of practical completion of the contract works.

Refer to the general section 1237 WARRANTIES for additional requirements.

Requirements

### 1.6	QUALIFICATIONS

Installers to be experienced, competent trades people familiar with the materials and techniques specified.

### 1.7	NO SUBSTITUTIONS

Substitutions are not permitted to any of the specified systems, components and associated products listed in this section.

Performance

### 1.8	ENERGY EFFICIENCY

Maintain the energy efficiency requirements to NZBC H1/AS1: Energy efficiency, 2.0 Building thermal envelope.  Install to NZS 4218 for small buildings, to NZS 4243 for large buildings and to the Kingspan Kooltherm® K10 FM Soffit Board technical requirements.

## 2.	PRODUCTS

Materials

### 2.1	SOFFIT BOARD

Kingspan Kooltherm® K10 FM Soffit Board to AS/NZS 4859.1, comprising fibre free rigid thermoset insulation core with autohesively bonded facings on both sides, an upper tissue-based facing and a lower facing of highly reflective aluminium foil, manufactured under quality control systems approved to BS EN ISO 9001.

Components

### 2.2	MECHANICAL FIXINGS - CONCRETE SOFFITS

Use a minimum 11 appropriate insulation fasteners with a minimum 35mm head diameter, approved by Kingspan Kooltherm® K10 FM Soffit Board Technical Department.

### 2.3	FIXINGS - TIMBER BATTENS

Fixings approved by Kingspan Kooltherm® K10 FM Soffit Board Technical Department.

### 2.4	TAPES

75mm (preferred) or 48mm wide Venture Tape aluminium foil tape or other approved by Kingspan Kooltherm® K10 FM Soffit Board Technical Department.

## 3.	EXECUTION

Conditions

### 3.1	DELIVERY, STORAGE AND HANDLING

Take delivery of materials and goods and store on site and protect from damage.

Protect finished surfaces, edges and corners from damage.  Move/handle goods in accordance with manufacturer's requirements.  Do not store outdoors and do not use boards that have been allowed to get wet.  Reject and replace goods that are damaged or will not provide the required finish.

### 3.2	SUBSTRATE

Do not commence work until the substrate is dry, plumb, level and to the standard required by the sheet manufacturer's requirements.

Application - general

### 3.3	STANDARDS AND TOLERANCES

Ensure flat concrete roof (soffit) /concrete floor (soffit) is of an acceptable standard before soffit lining commences.

Refer to the general section 1270 CONSTRUCTION for general requirements.

### 3.4	CUTTING

Cutting to be carried out using either a fine toothed saw or a sharp knife to cut through the insulation and paper backing of the plasterboard, then snapping the board face down over a straight edge and cutting the paper facing of the plasterboard on the other side. Ensure accurate trimming to achieve close butting joints and continuity of insulation.

Application - directly to concrete soffits

### 3.5	FIX SOFFIT BOARD

Fix and fully restrain Kingspan Kooltherm® K10 FM Soffit Board to concrete soffit using a minimum 11 appropriate insulation fasteners with minimum head diameter of 35mm.  Distribute fasteners evenly over the whole area of the board allowing a minimum 40 mm penetration into a solid substrate.

Ensure 4 fasteners along each length, no less than 50mm, no more than 150mm from edge of board and 3 fasteners along the middle.

### 3.6	BOARD JOINTS

Kingspan Kooltherm® K10 FM Soffit Board joints to be staggered or squared and taped with a 75mm wide (preferred) or 48mm wide (minimum), Venture Tape aluminium foil tape or approved alternative to manufacturer's instructions.

Application - to timber battens/furring channels

### 3.7	BATTENS/FURRING CHANNELS

Place 50mm x 25mm battens/furring channels at 600mm centres to coincide with the edges/centres of the boards.  The battens/furring channels to be fixed to the soffit by the

use of a suitable fixing method e.g. shot-fire may be considered.

### 3.8	FIX SOFFIT BOARD

Fix Kingspan Kooltherm® K10 FM Soffit Board to the treated timber battens/furring channels using suitable fixings.  Place at maximum 300mm (maximum 200mm when using nails into timber) in rows not greater than 600mm apart.

### 3.9	BOARD JOINTS

Kingspan Kooltherm® K10 FM Soffit Board joints to be staggered or squared and taped with a minimum 48 mm wide appropriate aluminium foil tape to manufacturer's instructions.

Application - finishing

### 3.10	TAPING

Ensure the surface of the Kingspan Kooltherm® K10 FM Soffit Board is free of dust, dirt or oils and that the climate conditions are suitable for the tape being used.  Clean the surface with a dry cloth before application of the tape.

Tape board joints with a minimum 48mm wide foil tape over the centre of the join, to tape manufacturer's recommendations.  Do not stretch tape tightly.  Wipe tape firmly from the centre out with a plastic squeegee.

Protect board edges using foil tape lapped 48mm on to the underside of board, in the absence of other protection.  Alternately use a UPVC c-section.

Completion

### 3.11	REPLACE

Replace damaged sheets or elements.

### 3.12	CLEAN DOWN

Clean down completed surfaces to remove irregularities to the sheet manufacturer's requirements, to leave completely smooth and clean to the standard required for following trades.

### 3.13	REMOVE

Remove debris, unused materials and elements from the site.

## 4.	SELECTIONS

For further details on selections go to www.forman.co.nz.

Substitutions are not permitted to the following.

Materials

### 4.1	SOFFIT BOARD

Location/substrate:	 ~/~

Type/ product:	Kingspan Kooltherm® K10 FM Soffit Board

Fixing method:	Mechanical Fastening

Size:	2270mm x 1200mm (2.72m²)